waveform_gen.so — LabVIEW CLFN Wiring Guide

Deploy

- Copy waveform_gen.so to the cRIO-9040, e.g.:
/home/lvuser/natinst/waveform_gen.so
or any path you prefer — just match it in the CLFN path string.

Call Library Function Node Setup (LabVIEW block diagram)

- Library path: /home/lvuser/natinst/waveform_gen.so (string constant)

- Function name: generatewaveform (must match exactly, case-sensitive)

- Calling convention: C

- Thread: Run in UI thread (or Any Thread for RT determinism)

- Return type: void

Terminal wiring — left to right, top to bottom

| # | Name | LV Control Type | Direction | Pass Mode | C Type |
| --- | --- | --- | --- | --- | --- |
| 1 | amp | DBL Numeric | Input | Value | double |
| 2 | freq_start | DBL Numeric | Input | Value | double |
| 3 | freq_stop | DBL Numeric | Input | Value | double |
| 4 | dt | DBL Numeric | Input | Value | double |
| 5 | length_pts | I32 Numeric | Input | Value | int32 |
| 6 | out_array | 1D DBL Array | Output | Array Data Ptr | double * |

Out-array wiring (most common confusion point)

- Pre-allocate a 1D DBL array of size length_pts BEFORE the CLFN call.

  - Use: Initialize Array → DBL, size = length_pts

- Wire that pre-allocated array to terminal 6 (Array Data Pointer).

- Wire the same terminal 6 output to your Waveform Graph / downstream node.

- The function writes in-place; LabVIEW owns the memory.

Recommended front-panel test values

| Control | Value | Reason |
| --- | --- | --- |
| amp | 2.0 | Visible on ±2 scale |
| freq_start | 10.0 | 10 Hz target |
| freq_stop | 10.0 | Same = fixed sine; change for sweep |
| dt | 0.001 | 1 kHz sample rate, 100 samples/cycle@10 Hz |
| length_pts | 1000 | 1-second record |

Common errors

| Symptom | Cause | Fix |
| --- | --- | --- |
| Flat line / zeros | Array not pre-allocated | Add Initialize Array before CLFN |
| Distorted waveform | dt too large (Nyquist violation) | dt < 1/(2*max_freq) |
| Error 1097 | Wrong function name spelling | Must be generatewaveform (lowercase) |
| Crash / segfault | out_array pointer NULL | Always pre-allocate; never pass empty |